# LAB-01

# ESD-I

# Please visit lab website-

# https://labs.dese.iisc.ac.in/embeddedlab/how-to-do/

# (or)

- Download Code Compose Studio windows-download

Linux- download

update system and Install Dependent libraries.

  - update system: sudo apt update

  - install dependent libraries:

- sudo apt install libc6-i386 libusb-0.1-4 libgconf-2-4 libncurses5 libpython2.7 libtinfo5

  - install CCS

  - if CCS was installed as user then run the driver install script

    - go to <CCS_INSTALL_DIR>/ccs/install_scripts

    - sudo ./install_drivers.sh

- Once download finished unzip the CCS_Setup and Run.

- This will start CCS Install Wizard.

- Accept the software License Agreement and click Next.

- choose installation typeselect the components TM4C123X....

- select debug probes.. and finish installation.

- Download TIVAWARE C Series

- (or)

- Download

- Create a New Project

To Create new project	Select -> Select New CCS project

->Select Compiler version, Target, connectioon as follows:

> in Help install GCC Arm Compiler tools.code composer studio includes a Linaro distribution of the ARM GNU compiler, so Programmers have the option of using GCC Compiler insted of the TI compiler to build their projects.

## Add path Build Variables:

Path variable –

when you add (link) a file to your project, you can specify a “relative to” path. The default is PROJECT_LOC which means that your linked resource (like a .lib file) will be linked relative to your project directory.

## Build variable –

used for items such as the search path for include files associated with a library – i.e. it is used when you build your project.

Variables can either have a project scope (that they only work for this project) or a WORKSPACE scope (that they work across all projects in the workspace).

In the next step, we need to add (link) a library file and then add a search path for include files. First, we’ll add these variables MANUALLY as PROJECT variables. Later, we will show you a quick and easy way to add these variables into your WORKSPACE so that any project in your workspace can use the variables.

# adding path variable:

->click New

->Type Tivaware_Install for the Name

->For the location ,Click the Folder and navigate to Tivaware Installation.

->click apply and close.

# Adding a Build Variable:

->click build Variable tab

-> select Tivaware_install

-> apply to all configurations.

->change Type to “Directory” and click “Ok”

Add the Include Search Paths:

->In Properties click Build->GNU Compiler->Directories:

->click “+” sign add the variable(Tivaware_Install).

->click ok.

Readme first-

https://www.ti.com/lit/ug/spmu296/spmu296.pdf?ts=1673930434318&ref_url=https%253A%252F

%252Fwww.google.com%252F

Data sheet download for Tiva c series User Manual -Tiva c series

Add files to your Project:

select File->New->Source File

copy the following code and paste it into the newly created main.c file: Blinky program:

#include <stdint.h>

#include "inc/tm4c123gh6pm.h" void delayMs(int n);

int main(void)

{	SYSCTL_RCGC2_R |= 0x00000020;	/* enable clock to GPIOF at clock gating control register */

GPIO_PORTF_DIR_R = 0x0E;	/* enable the GPIO pins for the LED (PF3, 2 1) as output */

GPIO_PORTF_DEN_R = 0x0E;	/* enable the GPIO pins for digital function

*/

while(1) {

GPIO_PORTF_DATA_R = 0x0E;	/* turn on all LEDs */

delayMs(500);

GPIO_PORTF_DATA_R = 0;	/* turn off all LEDs */

delayMs(500);

}

}

/* delay n milliseconds (16 MHz CPU clock) */

void delayMs(int n)

{

int i, j;

for(i = 0 ; i < n; i++)

for(j = 0; j < 3180; j++) {}	/* do nothing for 1 ms */

}

->Build your project &fix any errors

->Load Debug and Run